MeshCore GUI

Threaded BLE Edition

Design Document v3.1

PE1HVH

February 2026

# 1. Introduction

MeshCore GUI is a graphical user interface for MeshCore mesh network devices. The application communicates via Bluetooth Low Energy (BLE) with a MeshCore companion device and offers real-time monitoring and messaging functionality.

# 2. Architecture

The application uses a threaded architecture to separate BLE communication from the GUI:

## 2.1 Threading Model

| Component | Description |
| --- | --- |
| Main Thread | NiceGUI web interface, UI rendering and user interaction |
| BLE Thread | Separate thread with own asyncio event loop for BLE communication |
| SharedData | Thread-safe data container with threading.Lock for synchronization |
| Command Queue | Queue for GUI to BLE communication (send message, refresh, etc.) |

## 2.2 Data Flow

BLE Worker receives data via events and writes to SharedData. GUI reads a snapshot every 500ms and only updates changed elements. Commands from GUI to BLE go via a thread-safe queue.

## 2.3 Project Structure

| File | Class | Protocol | Responsibility |
| --- | --- | --- | --- |
| config.py | (none) | (none) | Constants and debug helper |
| protocols.py | (Protocol classes) | (defines all) | Interface contracts (DIP/ISP) |
| shared_data.py | SharedData | (implements all) | Thread-safe data store |
| ble_worker.py | BLEWorker | SharedDataWriter | BLE communication thread |
| main_page.py | DashboardPage | SharedDataReader | Dashboard UI (render + update) |
| route_builder.py | RouteBuilder | ContactLookup | Route data construction (pure logic) |
| route_page.py | RoutePage | SharedDataReadAndLookup | Route visualization UI |

# 3. Components

## 3.1 Protocol Interfaces

All inter-component dependencies go through typing.Protocol interfaces defined in protocols.py. This follows the Dependency Inversion Principle: consumers depend on abstractions, not on the concrete SharedData class. Only the composition root (meshcore_gui.py) knows the concrete implementation.

| Protocol | Consumer | Methods |
| --- | --- | --- |
| SharedDataWriter | BLEWorker | set_status, set_connected, set_contacts, set_channels, add_message, add_rx_log, update_from_appstart, update_from_device_query, get_next_command, get_contact_name_by_prefix |
| SharedDataReader | DashboardPage | get_snapshot, clear_update_flags, mark_gui_initialized, put_command |
| ContactLookup | RouteBuilder | get_contact_by_prefix |
| SharedDataReadAndLookup | RoutePage | SharedDataReader + ContactLookup combined |

## 3.2 SharedData Class

Thread-safe container for all shared state between BLE worker and GUI. Contains device info, contacts, channels, messages and RX log. Uses threading.Lock for all read/write operations. Implements all four Protocol interfaces.

| Attribute | Type / Description |
| --- | --- |
| name, public_key | str - Device identification |
| radio_freq, radio_sf, radio_bw | float/int - Radio configuration |
| contacts | Dict - {pubkey: {adv_name, type, adv_lat, adv_lon}} |
| channels | List[Dict] - [{idx, name}, ...] |
| messages | List[Dict] - Messages with time, sender, text, channel, direction, path_len, sender_pubkey |
| *_updated flags | bool - Initially True, cleared after GUI update |

## 3.3 BLEWorker Class

Handles all BLE communication in a separate thread. Connects to MeshCore device, subscribes to events, and processes commands from the GUI. Depends on SharedDataWriter protocol.

| Method | Function |
| --- | --- |
| _connect() | Connect to device, subscribe events, load data |
| _load_data() | Load device info, channels (hardcoded), contacts |
| _handle_command() | Process send_message, send_dm, send_advert, refresh |
| _on_channel_msg() | Callback for received channel messages |
| _on_contact_msg() | Callback for DM, lookup sender via pubkey_prefix |

## 3.4 DashboardPage Class

NiceGUI web interface with real-time dashboard. Update timer of 500ms for data refresh. Marks GUI as initialized only when channels AND contacts are loaded. Depends on SharedDataReader protocol.

## 3.5 RoutePage Class

Standalone page opened in a new tab when clicking a message. Shows hop count, Leaflet map with route polyline, and details table. Depends on SharedDataReadAndLookup protocol.

## 3.6 RouteBuilder Class

Pure data logic (no UI code). Constructs a route dictionary describing the path a message has taken through the mesh (sender, repeaters, receiver). Depends on ContactLookup protocol.

# 4. UI Layout

The interface consists of three columns:

| Column | Panels | Function |
| --- | --- | --- |
| Left (w-64) | Device, Contacts | Device info, clickable contacts for DM |
| Middle (flex) | Map, Input, Filter, Messages | Leaflet map, send message, channel filters |
| Right (w-64) | Actions, RX Log | Refresh/Advert buttons, RX log table |

# 5. Functionality

## 5.1 Channel Messages

Send messages to channels via dropdown selection and Send button. Received messages are filtered via checkboxes (DM + configured channels). Newest messages at top. Each message is clickable to open route visualization.

## 5.2 Direct Messages (DM)

Clicking on a contact in the list opens a dialog. Sending via mc.commands.send_msg(pubkey, text). Sender lookup via pubkey_prefix in contacts dictionary.

## 5.3 Map

Leaflet map with markers for own position (adv_lat/adv_lon) and contacts with location (adv_lat != 0 or adv_lon != 0). Centers on own position if available.

## 5.4 Route Visualization

Opens in a new browser tab for a selected message. Shows: hop count with SNR, Leaflet map with sender/repeater/receiver markers and polyline, and a route details table. Route data is built by RouteBuilder using out_path from the sender contact record.

# 6. Configuration

| Setting | Description |
| --- | --- |
| DEBUG | Set to True in config.py for verbose logging |
| CHANNELS_CONFIG | List of channels in config.py (hardcoded, BLE get_channel unreliable) |
| BLE Address | Command line argument to meshcore_gui.py |

# 7. Version History

| Version | Date | Changes |
| --- | --- | --- |
| 1.0 | 2026-01 | Initial monolithic implementation |
| 2.0 | 2026-02 | Threaded architecture, SharedData, NiceGUI |
| 3.0 | 2026-02 | Modular package, route visualization, flat structure |
| 3.1 | 2026-02 | Protocol interfaces (DIP/ISP), SOLID analysis |